### MCQ Revision On Lectures 1to 4

1. Logic microoperations operate on:
A) Decimal digits
B) Whole words only
C) Individual bits
D) Floating numbers
Answer: C

2. How many logic functions are possible with two binary variables?
A) 4
B) 8
C) 16
D) 32
Answer: C

3. The AND microoperation is represented as:
A) A + B
B) A • B
C) A ⊕ B
D) A’
Answer: B

4. XOR operation is represented by:
A) +
B) •
C) ⊕
D) ’
Answer: C

5. The operation F ← 0 is called:
A) Set
B) Transfer
C) Clear
D) Complement
Answer: C

6. F ← 1 is called:
A) Reset
B) Set to all 1’s
C) Clear
D) Mask
Answer: B

7. F ← A is called:
A) Complement
B) Transfer A
C) Mask
D) Clear
Answer: B

8. F ← A’ means:
A) AND
B) OR
C) Complement A
D) XOR
Answer: C

9. NAND operation equals:
A) (A + B)’
B) (A • B)’
C) A ⊕ B
D) A’ + B
Answer: B

10. NOR operation equals:
A) (A + B)’
B) (A • B)’
C) A + B
D) A ⊕ B
Answer: A

11. Selective set is performed by:
A) A • B
B) A + B
C) A ⊕ B
D) A • B’
Answer: B

12. Selective complement uses:
A) AND
B) OR
C) XOR
D) NOT
Answer: C

13. Selective clear operation is:
A) A • B
B) A + B
C) A • B’
D) A ⊕ B
Answer: C

14. Mask operation uses:
A) A + B
B) A • B
C) A ⊕ B
D) A’
Answer: B

15. Compare operation uses:
A) A + B
B) A • B
C) A ⊕ B
D) A’ + B
Answer: C

16. Insert operation requires:
A) Only OR
B) Only AND
C) Mask then OR
D) XOR only
Answer: C

17. In logical shift, serial input is:
A) 1
B) Previous bit
C) 0
D) Sign bit
Answer: C

18. RTL notation for logical left shift:
A) shr
B) shl
C) cir
D) ashl
Answer: B

19. RTL notation for logical right shift:
A) shr
B) shl
C) cil
D) ashr
Answer: A

20. In circular shift, serial input comes from:
A) 0
B) 1
C) Sign bit
D) Bit shifted out
Answer: D

21. Circular left shift notation:
A) cir
B) cil
C) shl
D) ashl
Answer: B

22. Arithmetic shifts are used for:
A) Unsigned numbers
B) Signed numbers
C) Characters
D) Logic only
Answer: B

23. Arithmetic left shift multiplies by:
A) 1
B) 2
C) 4
D) 8
Answer: B

24. Arithmetic right shift divides by:
A) 2
B) 4
C) 8
D) 10
Answer: A

25. Arithmetic shift must preserve:
A) LSB
B) MSB
C) Sign bit
D) Carry
Answer: C

26. Overflow in arithmetic left shift occurs when:
A) MSB = 0
B) LSB = 1
C) Two leftmost bits differ before shift
D) Carry = 1
Answer: C

27. RTL for arithmetic right shift:
A) shl
B) ashl
C) shr
D) ashr
Answer: D

28. F = A + B is:
A) Subtraction
B) Addition
C) AND
D) OR
Answer: B

29. F = A + 1 performs:
A) Decrement
B) Increment
C) Complement
D) Clear
Answer: B

30. F = A - 1 performs:
A) Increment
B) Decrement
C) Transfer
D) Clear
Answer: B

31. F = A + B’ + 1 equals:
A) Addition
B) Subtraction
C) AND
D) OR
Answer: B

32. F = A ⊕ B belongs to:
A) Arithmetic circuit
B) Logic circuit
C) Shift circuit
D) Memory
Answer: B

33. When S3 = 1 and S2 = 0, operation is:
A) Logic
B) Arithmetic
C) Shift right
D) Shift left
Answer: C

34. When S3 = 1 and S2 = 1, operation is:
A) Addition
B) XOR
C) Shift left
D) OR
Answer: C

35. Addressing modes specify:
A) Data size
B) Operand location
C) Instruction length
D) Clock cycle
Answer: B

36. Immediate addressing means:
A) Data in register
B) Data in memory
C) Data in instruction
D) Data in ALU
Answer: C

37. Direct addressing provides:
A) Address of data
B) Data itself
C) Address of address
D) Register number
Answer: A

38. Indirect addressing provides:
A) Data
B) Register
C) Address of address
D) Constant
Answer: C

39. Register addressing stores operand in:
A) Memory
B) Cache
C) Register
D) Disk
Answer: C

40. Register indirect addressing uses:
A) Immediate value
B) Register holding address
C) Base only
D) Index only
Answer: B

41. Indexed addressing uses:
A) Base register
B) Index register as offset
C) Constant only
D) Immediate mode
Answer: B

42. Based addressing uses:
A) Index register
B) Base register
C) ALU
D) Cache
Answer: B

43. Effective address is:
A) Opcode
B) Operand value
C) Actual memory location of operand
D) Instruction size
Answer: C

44. Indexed addressing calculates EA by:
A) Subtracting index
B) Adding index to address field
C) Dividing
D) Complementing
Answer: B

45. Base addressing calculates EA by:
A) Adding displacement to base
B) XOR
C) AND
D) Shifting
Answer: A

46. XOR with 1 results in:
A) Same bit
B) Complemented bit
C) Zero
D) One
Answer: B

47. Masking is mainly used to:
A) Add bits
B) Clear selected bits
C) Multiply
D) Divide
Answer: B

48. Circular shift differs from logical shift because it:
A) Uses sign bit
B) Inserts 0
C) Reuses shifted-out bit
D) Clears register
Answer: C

49. Arithmetic right shift keeps:
A) LSB constant
B) Sign bit constant
C) Carry bit
D) All bits zero
Answer: B

50. Microoperations operate at:
A) High-level language
B) Assembly level
C) Register level
D) User level
Answer: C

51.

A = 1011
B = 0101
Perform Selective Set (A ← A + B). Result?
A) 1111
B) 1011
C) 0101
D) 1000
Answer: A

52.
A = 1011
B = 0101
Perform Selective Clear (A ← A • B’). Result?
A) 1011
B) 1010
C) 1000
D) 0010
Answer: B

53.
A = 1001
B = 1100
Perform Mask (A ← A • B).
A) 1000
B) 1100
C) 0100
D) 0001
Answer: A

54.
A = 1110
B = 0101
Perform Compare (A ← A ⊕ B).
A) 1011
B) 1111
C) 0101
D) 1001
Answer: A

55.
If A = 1111, what is (A • A’)?
A) 1111
B) 0000
C) 1010
D) 0101
Answer: B

56.
Which operation can detect unequal bits between A and B?
A) AND
B) OR
C) XOR
D) NOR
Answer: C

57.
How many logic functions exist for 3 binary variables?
A) 8
B) 16
C) 64
D) 256
Answer: D

58.
R = 10110000
After shl R, result is:
A) 01100000
B) 01100001
C) 10110000
D) 01011000
Answer: A

59.
R = 10110000
After shr R, result is:
A) 01011000
B) 00101100
C) 01011001
D) 11011000
Answer: A

60.
R = 10011001
After circular left shift (cil), result is:
A) 00110011
B) 00110010
C) 10011001
D) 11001100
Answer: A

61.
R = 10011001
After circular right shift (cir), result is:
A) 11001100
B) 01001100
C) 00110011
D) 10011001
Answer: A

62.
Signed 8-bit number: 01001000
After arithmetic left shift, value becomes:
A) 10010000
B) 00100100
C) 01001000
D) 00010010
Answer: A

63.
Signed 8-bit number: 11001000
After arithmetic right shift, result is:
A) 01100100
B) 11100100
C) 00100100
D) 01001000
Answer: B

64.
Overflow in arithmetic left shift occurs when:
A) LSB = 1
B) Carry = 1
C) Two MSBs differ before shift
D) Result = 0
Answer: C

65.
Arithmetic right shift mainly performs:
A) Multiply by 2
B) Divide by 2
C) Clear register
D) Complement
Answer: B

66.
If S1S0 = 00 and Cin = 0, operation is:
A) A + B
B) A + 1
C) A - 1
D) A’

Answer= A

67.
If F = A + B’ + 1, the operation equals:
A) Addition
B) Subtraction
C) AND
D) XOR
Answer: B

68.
If Cin=1,  S1S0=11 → Operation?
A) OR
B) AND
C) Transfer A
D) Complement
Answer: C

69.
A = 0101, B = 0011
Perform A + B. Result?
A) 1000
B) 0111
C) 1111
D) 0100
Answer: A

70.
Insert 0011 into lower 4 bits of 11110000. Result?
A) 11110011
B) 11111111
C) 00000011
D) 11110000
Answer: A

71.
Mask 11001100 with 11110000.
A) 11000000
B) 00001100
C) 11111100
D) 10000000
Answer: A

72.
Selective complement 10101010 with 00001111.
A) 10100101
B) 10100000
C) 01010101
D) 10101010
Answer: A

73.
Instruction: LOAD #50
Addressing mode?
A) Direct
B) Immediate
C) Register
D) Indirect
Answer: B

74.
Instruction: LOAD 500
(500 contains 25)
Mode?
A) Immediate
B) Direct
C) Register
D) Indexed
Answer: B

75.
Instruction: LOAD @500
(500 contains 800, 800 contains 30)
Loaded value?
A) 500
B) 800
C) 30
D) 25
Answer: C

76.
If R1 = 1000 and instruction uses register addressing, operand is:
A) 1000
B) Memory[1000]
C) Address of R1
D) 0
Answer: A

77.
Indexed mode: Address field = 500, Index = 20
Effective address = ?
A) 480
B) 500
C) 520
D) 5200
Answer: C

78.
Base register = 1000, displacement = 50
Effective address = ?
A) 950
B) 1050
C) 1000
D) 50
Answer: B

79.
XOR with 1 results in:
A) Same bit
B) Complement
C) Zero
D) One
Answer: B

80.
AND with 0 results in:
A) 1
B) Same bit
C) 0
D) Complement
Answer: C

81.
OR with 1 results in:
A) 1
B) 0
C) Same bit
D) Complement
Answer: A

82.
Logical shift differs from arithmetic shift because:
A) Logical preserves sign
B) Arithmetic preserves sign
C) Logical multiplies
D) Arithmetic clears bits
Answer: B

83.
Circular shift is useful for:
A) Clearing register
B) Bit rotation
C) Complement
D) Masking
Answer: B

84.
Which operation can be used to test if two registers are identical?
A) AND
B) OR
C) XOR
D) NAND
Answer: C

85.
If result of A ⊕ B = 0000, then:
A) A = 0
B) B = 0
C) A = B
D) A = 1
Answer: C

86.
Arithmetic right shift of negative number keeps it:
A) Positive
B) Zero
C) Negative
D) Complemented
Answer: C

87.
Masking with 0000 results in:
A) Same value
B) All 1s
C) All 0s
D) Complement
Answer: C

89.
The difference between indexed and based addressing is:
A) One uses memory
B) One uses ALU
C) Index = offset, Base = starting address
D) No difference
Answer: C

90.
In arithmetic subtraction using ALU, B’ + 1 represents:
A) AND
B) Two’s complement
C) XOR
D) OR
Answer: B

91.
If MSB changes after arithmetic shift left, this may indicate:
A) Clear
B) Compare
C) Overflow
D) Mask
Answer: C

91.
Immediate addressing is fastest because:
A) Uses cache
B) No memory access needed
C) Uses register
D) Uses index
Answer: B

92.
Register indirect addressing requires:
A) Two memory accesses
B) One memory access
C) No memory access
D) Three accesses
Answer: A

# Case-Based Scenario Questions

### Case 1

A = 11001100
B = 00001111

Two operations are executed sequentially:

- A ← A • B

- A ← A ⊕ B

1. What is the final value of A?

A) 00000000
B) 00001111
C) 11110000
D) 11000011

Answer: B

### Case 2

A signed 8-bit register contains:

A = 10010000

An arithmetic right shift is performed twice.

2. What is the final value?

A) 00100100
B) 11001000
C) 11100100
D) 01001000

Answer: C

### Case 3

Memory contents:

| Address | Value |
| --- | --- |
| 400 | 900 |
| 900 | 1200 |
| 1200 | 55 |

Instruction: LOAD 400

3. What value is loaded into the accumulator for indirect addressing ?

A) 400
B) 900
C) 1200
D) 55

Answer: C

### Case 4

Base Register = 1000
Index Register = 50
Address field in instruction = 200

Instruction uses indexed addressing.

4. Effective address = ?

A) 1200
B) 1050
C) 250
D) 1150

Answer: C
(EA = 200 + 50)

### Case 5

A = 11110000

We want to insert 1010 into the lower 4 bits.

Which sequence is correct?

A) OR then AND
B) AND with mask 11110000 then OR with 00001010
C) XOR then AND
D) Complement then OR

Answer: B

### Case 7

A processor performs:

F = A + B’ + 1

A = 0101
B = 0011

7. Result F = ?

A) 0010
B) 0110
C) 1000
D) 0001

Answer: A
(5 – 3 = 2)

### Case 8

R = 01111111

An arithmetic left shift is performed.

8. What happens?

A) Result = 11111110, no issue
B) Result = 11111110, overflow occurs
C) Result = 00111111
D) No change

Answer: B

### Case 9

Two registers are compared using XOR.
Result = 00000000

9. What does this indicate?

A) Both are zero
B) Both are negative
C) Registers are identical
D) Overflow occurred

Answer: C

### Case 10

A = 10101010

A circular right shift is performed.

10. Result = ?

A) 01010101
B) 11010101
C) 01010100
D) 10101010

Answer: A

11. Why is subtraction implemented as A + B’ + 1 in ALU?
A) Saves memory
B) Uses two’s complement representation
C) Avoids overflow
D) Makes addition faster
Answer: B

12. Why must arithmetic right shift preserve MSB?
A) To maintain magnitude
B) To avoid clearing bits
C) To preserve sign of signed number
D) To detect overflow
Answer: C

13. Why is immediate addressing usually faster than direct addressing?
A) Uses cache
B) No memory access needed for operand
C) Uses register
D) Uses ALU
Answer: B

14. Which addressing mode is best for array access?
A) Immediate
B) Indexed
C) Register
D) Indirect
Answer: B

15. In register indirect addressing, why are two memory accesses needed?
A) For opcode
B) First to get address, second to get operand
C) For shift
D) For increment
Answer: B

16. Why does XOR detect bit differences?
A) Outputs 1 when bits are equal
B) Outputs 1 when bits differ
C) Always outputs 0
D) Always outputs 1
Answer: B

17. Logical shift right on signed negative numbers is dangerous because:
A) It multiplies
B) It preserves sign
C) It destroys sign information
D) It clears register
Answer: C

19. If A + B = A, what must be true?
A) B has 1s where A has 1s
B) B has 0s where A has 0s
C) B has 0s in all positions where A has 0
D) B = 0
Answer: C

20. In an arithmetic left shift, overflow happens when:
A) MSB changes
B) Carry is 1
C) Two highest bits differ before shift
D) LSB is 1
Answer: C

21. Why is circular shift useful in cryptographic or hashing algorithms?
A) Clears bits
B) Preserves all bits while changing positions
C) Removes sign
D) Detects overflow
Answer: B

22. What is the main conceptual difference between base and indexed addressing?
A) One uses ALU
B) One uses memory
C) Base holds starting address, index holds offset
D) No difference
Answer: C

23. Why is masking important in hardware control registers?
A) For addition
B) For selective modification of bits
C) For division
D) For multiplication
Answer: B

24. Which operation can toggle only selected bits?
A) AND
B) OR
C) XOR
D) NOR
Answer: C

### 1. A register is:

A) A memory chip
B) A storage location inside CPU
C) A control signal
D) A type of bus

Answer: B

### 2. A microoperation is performed in:

A) One instruction cycle
B) One clock pulse
C) One second
D) Multiple clock cycles always

Answer: B

### 3. RTL stands for:

A) Register Timing Logic
B) Register Transfer Language
C) Random Transfer Logic
D) Register Test Level

Answer: B

### 4. The symbol "←" in RTL means:

A) Compare
B) Equal
C) Transfer
D) Add

Answer: C

### 5. In R2 ← R1, the source register is:

A) R2
B) R1
C) Both
D) None

Answer: B

### 6. In R2 ← R1, the destination register is:

A) R1
B) R2
C) BUS
D) MAR

Answer: B

### 7. Register transfer is:

A) Destructive to source
B) Non-destructive to source
C) Deletes source
D) Clears both registers

Answer: B

### 8. The BUS is:

A) A storage device
B) A control unit
C) A group of wires for data transfer
D) A memory register

Answer: C

### 9. The main purpose of a bus is to:

A) Increase clock speed
B) Reduce number of direct connections
C) Store instructions
D) Replace memory

Answer: B

### 10. A multiplexer (MUX) in BUS is used to:

A) Select destination register
B) Select source register
C) Store data
D) Increase memory size

Answer: B

### 11. A decoder in BUS is used to:

A) Select source register
B) Select destination register
C) Add numbers
D) Shift bits

Answer: B

### 12. If n registers are fully connected directly, required connections are:

A) n
B) 2n
C) n(n−1)
D) n²/2

Answer: C

### 13. The cost of direct register connection is:

A) O(n)
B) O(log n)
C) O(n²)
D) O(1)

Answer: C

### 14. The control function P: R2 ← R1 means:

A) Always transfer
B) Transfer only if P = 1
C) Transfer if P = 0
D) Clear R2

Answer: B

### 15. Comma in RTL indicates:

A) Addition
B) Sequence
C) Simultaneous operations
D) Subtraction

Answer: C

### 16. In BUS ← R1, R2 ← BUS:

A) Bus is implicit
B) Bus is explicit
C) No transfer occurs
D) Memory is used

Answer: B

### 17. MAR stands for:

A) Memory Access Register
B) Main Address Register
C) Memory Address Register
D) Micro Address Register

Answer: C

### 18. PC stands for:

A) Program Counter
B) Primary Counter
C) Processing Control
D) Program Code

Answer: A

### 19. IR stands for:

A) Internal Register
B) Instruction Register
C) Input Register
D) Immediate Register

Answer: B

### 20. Memory with r = 2^k words requires:

A) r address lines
B) k address lines
C) n address lines
D) 2k address lines

Answer: B

### 21. Memory read operation in RTL is:

A) M ← DR
B) DR ← M
C) M ← AR
D) AR ← DR

Answer: B

### 22. Memory write operation in RTL is:

A) DR ← M
B) M ← DR
C) AR ← DR
D) DR ← AR

Answer: B

### 23. During memory read:

A) Write signal = 1
B) Read signal = 1
C) Both signals = 1
D) No control signal needed

Answer: B

### 24. A register can be divided into:

A) Subfields
B) Bits
C) Portions
D) All of the above

Answer: D

### 25. The clock controls:

A) Data value
B) Timing of microoperations
C) Address size
D) Bus width

Answer: B

### 26. Only one register can place data on the bus at a time to avoid:

A) Overflow
B) Underflow
C) Data collision
D) Shift error

Answer: C

### 27. A MUX has:

A) Multiple inputs and one output
B) One input and multiple outputs
C) No inputs
D) Only memory lines

Answer: A

### 28. A decoder activates:

A) All registers
B) Exactly one output line
C) The bus
D) The clock

Answer: B

### 29. R3 ← R5 happens:

A) Bit by bit sequentially
B) Simultaneously for all bits
C) Only high bits
D) Only low bits

Answer: B

### 30. The main advantage of using a bus is:

A) Faster memory
B) Reduced hardware complexity
C) Larger registers
D) More clock cycles

Answer: B

## Case 1

A system has 4 registers (R0–R3) connected to a common bus using multiplexers.
The control unit selects R2 as the source and R1 as the destination.

### 1. What does the MUX do in this case?

A) Enables R1 to load
B) Places R2 on the bus
C) Clears R2
D) Increments R1

Answer: B

### 2. What does the decoder do?

A) Selects R2 as source
B) Sends address to memory
C) Enables R1 load input
D) Clears bus

Answer: C

### 3. The correct RTL expression is:

A) R2 ← R1
B) BUS ← R1
C) R1 ← R2
D) M ← R2

Answer: C

## Case 2

A system tries to allow R1 and R2 to place data on the bus at the same time.

### 4. What will happen?

A) Faster transfer
B) Data collision
C) Automatic selection
D) No effect

Answer: B

### 5. Why must only one register drive the bus at a time?

A) To reduce memory size
B) To avoid hardware damage/conflict
C) To increase clock speed
D) To simplify decoder

Answer: B

## Case 4

Given the RTL:

P: R3 ← R5, MAR ← IR

P = 1.

### 8. When do both transfers occur?

A) Different clock cycles
B) Before clock
C) Same clock pulse
D) After memory read

Answer: C

### 9. If P = 0, what happens?

A) Only R3 loads
B) Only MAR loads
C) Both load
D) No transfer

Answer: D

## Case 5

Given:

R1 ← M[MAR]

### 10. What must be active?

A) Write = 1
B) Read = 1
C) Load = 0
D) Decoder disabled

Answer: B

## Case 6

Given:

M[MAR] ← R4

### 12. What type of operation is this?

A) Memory read
B) Memory increment
C) Memory write
D) Register transfer

Answer: C

## Case 7

A 16-register system uses a bus architecture.

### 14. How many select lines are required for the MUX?

A) 2
B) 3
C) 4
D) 16

(2⁴ = 16 → need 4 select lines)

Answer: C

### 15. Why is a decoder needed in bus architecture?

A) To reduce clock speed
B) To select which register loads from bus
C) To store data
D) To perform arithmetic

Answer: B

## 🔹 Question 1

- 4 registers (R0–R3)

- All connected to a 4×1 MUX

- MUX output connected to BUS

- Decoder connected to Load inputs of R0–R3

- Select lines S1 S0 = 10

- Decoder output D1 = 1

### What transfer occurs?

A) R1 ← R2
B) R2 ← R1
C) R2 ← R3
D) R1 ← R3

Analysis:
S1S0 = 10 → selects R2 as source
D1 = 1 → enables R1 as destination

Answer: A

## 🔹 Question 2

- 8 registers connected to bus

- 8×1 MUX selects source

- Select lines are 011

- Decoder enables R5

### What happens?

A) R3 ← R5
B) R5 ← R3
C) R5 ← R7
D) R7 ← R5

Analysis:
011 = 3 → R3 selected as source
Decoder enables R5 → destination

Answer: B

## 🔹 Question 4

- MAR connected to memory address lines

- Read = 1

- Memory output connected to bus

- Decoder enables R2

### What RTL operation is occurring?

A) M ← R2
B) R2 ← M[MAR]
C) MAR ← R2
D) R2 ← MAR

Answer: B

## 🔹 Question 5

- MAR connected to memory address lines

- Write = 1

- R3 connected to bus

- Bus connected to memory data input

### What operation is occurring?

A) R3 ← M[MAR]
B) M[MAR] ← R3
C) MAR ← R3
D) R3 ← MAR

Answer: B

## 🔹 Question 6

- 16 registers connected via bus

- MUX uses 4 select lines

- Decoder uses 4 input lines

### Why are 4 select lines needed?

A) 2² = 4
B) 2³ = 8
C) 2⁴ = 16
D) 4² = 16

Answer: C

## 🔹 Question 7

- Two registers R1 and R2 both enabled to place data on bus simultaneously

### What is the result?

A) Correct transfer
B) Faster operation
C) Bus contention
D) Automatic priority selection

Answer: C

## 🔹 Question 8

Diagram description:

- BUS ← R6

- Decoder enables R4 and R5 simultaneously

- Clock pulse occurs

### What happens?

A) Only R4 loads
B) Only R5 loads
C) Both R4 and R5 load
D) Bus collision

Answer: C

## 🔹 Question 10

- BUS ← IR

- Decoder enables MAR

- Clock pulse

### What is happening?

A) Instruction fetch
B) Memory write
C) MAR ← IR
D) IR ← MAR

Answer: C

### Q1

Control signal yT2 performs:

R2 ← R1
R1 ← R2

Which hardware configuration correctly implements this in one clock cycle?

A) Single common bus without temporary register
B) Two buses with simultaneous load
C) One bus with sequential transfer
D) ALU-based swap using XOR only

Answer: B

### Q4

Given a 4-bit adder-subtractor:

M = 1
A = 0110
B = 1001

What is the final result (S3–S0)?

A) 1101
B) 0101
C) 1111
D) 1010

Answer: D

### Q5

For Q4, the carry out (C4) indicates:

A) Borrow occurred
B) Overflow occurred
C) No borrow
D) No arithmetic meaning

Answer: B

A system has 16 registers, each 8 bits wide.

### Q8

Number of multiplexers required:

A) 8
B) 16
C) 128
D) 32

Answer: A

### Q9

Size of each multiplexer:

A) 8×1
B) 16×1
C) 4×1
D) 2×1

Answer: B

### Q10

Number of selection lines per multiplexer:

A) 2
B) 3
C) 4
D) 8

Answer: B

Original A:

1111 0000 1010 1010

We want to insert:

0101

into the lowest 4 bits.

### Q11

The correct mask is:

A) 1111 0000 1010 0000
B) 1111 1111 1111 0000
C) 0000 0000 0000 1111
D) 1111 0000 0000 0000

Answer: B

### Q13

After masking, the intermediate value becomes:

A) 1111 0000 1010 0000
B) 1111 0000 1010 1111
C) 0000 0000 0000 0000
D) 1111 1111 1111 0000

Answer: A

### Q14

Final result after OR insertion:

A) 1111 0000 1010 0101
B) 1111 0000 1010 1010
C) 1111 0000 1010 1111
D) 0000 0000 0000 0101

Answer: A

### Q15

Signed 8-bit number:

01000001

After arithmetic left shift:

A) 10000010 with overflow
B) 10000010 without overflow
C) 00100000
D) 01000001

Answer:A

### Q16

Signed 8-bit number:

10100000

After arithmetic right shift twice:

A) 11101000
B) 00101000
C) 10101000
D) 01101000

Answer: A

### Q17

Arithmetic overflow occurs when:

A) Carry out = 1
B) MSB changes unexpectedly
C) LSB = 1
D) Result = 0

Answer:B

Memory:

| Address | Value |
| --- | --- |
| 500 | 900 |
| 900 | 1200 |
| 1200 | 77 |

### Q18

Instruction: LOAD 500
Value loaded for direct addressing mode:

A) 500
B) 900
C) 1200
D) 77

Answer: B

### Q19

Instruction: LOAD 500
Value loaded indirect addressing mode:

A) 900
B) 1200
C) 77
D) 500

Answer: B

Base register = 1000
Index register = 30
Address field = 200

### Q21

Indexed addressing effective address:

A) 1030
B) 230
C) 1200
D) 970

Answer: B

### Q22

Based addressing effective address:

A) 1030
B) 1200
C) 200
D) 1230

Answer: B

### Q25

In a 32M × 2 byte memory, number of address line, data lines required:

A) 24, 8
B) 25, 16
C) 26, 16
D) 32, 8

Answer: B